Call “PREFLIGHT CHECKLIST.”

PREFLIGHT PROCEDURE – CAPTAIN

The captain normally does this procedure. The first officer may do this procedure if needed.

SPEED BRAKE lever ..............................................................Armed

VHF communications radios .............................................................. Set

VHF navigations radios……………………………………………... Set

ADF ………………………………………………………………… Set

Transponder panel ...............................................................................Set

Set the altimeter. (C and FO)

YAW DAMPER switch – ON

GALLEY power switch – ON

APU switch  ….....................................................................START

CAUTION: Left center tank fuel pump switches should be positioned ON only if the fuel quantity in the center tank exceeds 1000 lbs. If not, switch ON a left tank pump.

APU GENERATOR bus switches – ON

Passenger signs ....................................................................................Set

NO SMOKING switch – AUTO or ON

FASTEN BELTS switch – AUTO or ON

EMERGENCY LIGHTS – ON -  Guard closed

WINDOW HEAT switches ................................................................ ON

Position the switches ON at least 10 minutes before takeoff.

ELECTRIC HYDRAULIC PUMPS switches – OFF

Air conditioning

PACK switches – Set

One switch – ON

Other switch – OFF

Engine BLEED air switches – ON

APU BLEED air switch – ON

FLIGHT ALTITUDE indicator – Cruise altitude

LANDING ALTITUDE indicator – Departure field elevation

CABIN ALTITUDE indicator – Default.

FLIGHT/GROUND switch – GRD

POSITION light switch – As needed

ANTI–COLLISION light switch – OFF

WING illumination switch – As needed

WEATHER RADAR panel .................................................................Set