# Shutdown Procedure

After the airplane has come to a complete stop, perform the following actions.

Parking brake .......................................................................................Set

Parking brake warning light –Illuminated

Electrical ................................................................................ ON

Verify APU powering busses.

Start levers ................................................................... CUTOFF

If possible, operate the engines at idle for three minutes prior to shutdown to thermally stabilize the engine hot sections. Operating times at or near idle, such as taxiing before shutdown, are applicable to this three–minute period. If operational requirements dictate, the engines may be shut down with a one–minute cooling period.

FASTEN BELTS switch ....................................................... OFF

ANTI–COLLISION light switch .......................................... OFF

FUEL PUMP switches ..............................................................................OFF

GALLEY power switch ............................................As required

WINDOW HEAT switches ........................................OFF

WING and ENGINE ANTI–ICE switches .......................OFF

ELECTRIC HYDRAULIC PUMP switches ...................OFF

RECIRCULATION FAN switch(es) ......................... As desired

Air conditioning PACK switches .......................... One AUTO Engine BLEED air switches.............................................. ON

APU BLEED air switch ..................................................................... ON

Exterior lights ......................................................... As required

AUTO BRAKE select switch..........................................OFF

Flight Deck lights ....................................................................As desired

SPEEDBRAKE lever ....................................... DOWN detent

WEATHER RADAR...................................................OFF

Transponder............................................................ As required

The captain calls “SHUTDOWN CHECKLIST.”

The first officer accomplishes the SHUTDOWN CHECKLIST.

# Secure

IRS mode selectors.............................................................OFF EMERGENCY EXIT lights switch ..................................OFF

Air conditioning PACK switches .......................................OFF

If APU was operating:

It is recommended that the APU be operated for one full minute with no pneumatic load prior to shutdown.

APU switch/GROUND POWER switch ................................ OFF

Delay approximately 20 seconds after APU shutdown for the APU door to close to assure the APU will start on the next flight.

BATTERY switch ................................................................. OFF

The captain calls “SECURE CHECKLIST.”

The first officer accomplishes the SECURE CHECKLIST.