# Go–Around Procedure

| PILOT FLYING |  |  |  |  |  | PILOT NOT FLYING |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Push TO/GA switch. Call “FLAPS 15.” If full GA thrust is required, push TO/GA switch again after reduced GA thrust is established. |  |  |  |  |  | Monitor N1 indication. Position FLAP lever to 15 and monitor flap retraction. |  |  |  |
| Confirm rotation to go–around attitude and monitor autopilot. |  |  |  |  |  |  |  |  |  |
| Verify mode annunciation. |  |  |  |  |  |  |  |  |  |
| When positive rate of climb is indicated, call “GEAR UP” and monitor acceleration. |  |  |  |  |  | Verify positive rate of climb. Position landing gear lever UP. |  |  |  |
| Check flight instrument indications (MCP speed window blanks.) |  |  |  |  |  |  |  |  |  |
| Call “TUNE RADIOS MISSED APPROACH.” |  |  |  |  | FOR | Tune radios as directed. |  |  |  |
| Above 400 feet, select appropriate roll mode and verify proper mode annunciation. |  |  |  |  |  | Observe mode annunciation. |  |  |  |
| Retract flaps on flap speed schedule. |  |  |  |  |  | Position FLAP lever as directed and monitor flaps and slats retraction. |  |  |  |
| Verify airplane levels off at selected altitude and maintains flap maneuvering speed. |  |  |  |  |  |  |  |  |  |
| Call “AFTER TAKEOFF CHECKLIST.” |  |  |  |  |  | Accomplish the TAKEOFF checklist. |  | AFTER |  |