DESCENT AND APPROACH PROCEDURE

| PILOT FLYING |  | PILOT NOT FLYING |  |  |
| --- | --- | --- | --- | --- |
| Check and set VREF and approach speeds as required. |  |  |  |  |
|  |  | Set anti–ice as required. |  |  |
|  |  | Verify pressurization is set for destination airport elevation and system operating normally. |  |  |
| Set AUTOBRAKE select switch to desired brake setting. |  |  |  |  |
| Set and crosscheck altimeters at transition level. |  |  |  |  |
| Set and crosscheck course selection and DH REF/radio altimeters as required for approach. |  |  |  |  |
| Set and verify ADF and VHF NAV radios for approach. |  |  |  |  |
|  |  | Position inboard landing lights ON passing through 10,000 feet. |  |  |
| Call “DESCENT–APPROACH CHECKLIST.” |  | Accomplish the DESCENT – APPROACH checklist. |  |  |
| Announce “FLAPS __” according to flap speed schedule. |  | Position FLAP lever as directed and monitor flap and slat extension. Accomplish standard callouts. |  |  |
| Approaching selected FMC altitude verify level off and mode annunciation. |  |  |  |  |

# Approach Procedure

Using flaps as speedbrakes is not recommended.

The following procedures are used for flap extension:

- Select flaps 1 when decelerating through the flaps–up maneuvering speed, displayed on the speed tape as a green “0.”

- When appropriate, select the next flap position and then set the airspeed cursor to that flap maneuver speed.

Note: Flap maneuver speeds provide approximately 15 to 20 knots above the minimum maneuvering speed for each flap setting.

Note: If performance requires use of flaps 15 for landing, place the GROUND PROXIMITY flap inhibit switch to FLAP INHIBIT.

If the flap maneuvering speeds cannot be displayed, reference the Performance In–flight section for speed schedules.

When on final approach in landing configuration, it is not recommended to set the A/T command speed to allow for wind or gust corrections. Through airspeed and acceleration sensing, the A/T corrects for normal wind gusts. Higher command speed settings result in excessive approach speeds. The recommended A/T approach speed setting is VREF + 5.

| FLAP MANEUVERING SCHEDULE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FLAP | AT & BELOW |  | ABOVE | ABOVE |  |
| POSITION | 117,000 LBS |  | 117,000 LBS | 138,500 LBS |  |
|  | (53,070 KGS) |  | (53,070 KGS) | (62,823 KGS) |  |
|  |  |  | UP TO (138,500 |  |  |
|  |  |  | LBS (62,823 |  |  |
|  |  |  | KGS) |  |  |
| FLAPS UP | 210 |  | 220 | 230 |  |
| FLAPS 1 | 190 |  | 200 | 210 |  |
| FLAPS 5 | 180 |  | 190 | 200 |  |
| FLAPS 10 | 170 |  | 180 | 190 |  |
| FLAPS 15 | 150 |  | 160 | 170 |  |
| FLAPS 25 | 140 |  | 150 | 160 |  |
| PILOT FLYING |  | PILOT NOT FLYING |  |  |  |
| When on localizer intercept heading, verify ILS tuned and identified, LOC and G/S pointer displayed, arm APP mode and engage the second autopilot. |  | Set transponder mode selector to desired TCAS mode. (TCAS equipped airplanes.) |  |  |  |
| Verify mode annunciation. |  |  |  |  |  |
| At localizer capture, verify proper mode annunciation and set appropriate heading. |  | Verify proper mode annunciation. |  |  |  |
| At glide slope “alive,” announce “GEAR DOWN,” “FLAPS 15." Arm the speed brakes and check green light illuminated. Call “LANDING CHECKLIST DOWN TO FLAPS.” |  | Position landing gear lever DN, and FLAP lever to the 15 detent. Position engine start switch to CONT. Check RECALL. |  |  |  |
|  |  | Accomplish the LANDING checklist down to flaps. State “HOLDING AT FLAPS.” |  |  |  |
| At glide slope capture, verify proper mode annunciation, check N1 cursor at the go–around limit and set missed approach altitude. |  |  |  |  |  |
| Call “FLAPS ” as required for landing. Set MCP speed selector at VREF + 5 kts. |  | Position FLAP lever as directed. |  |  |  |
| At final approach fix/OM, verify crossing altitude. |  |  |  |  |  |
| Call “COMPLETE THE LANDING CHECKLIST.” |  | Complete the LANDING checklist. |  |  |  |
| Monitor approach progress and guard the controls. At 500 feet AGL, verify FLARE is armed. At approximately 50 feet AGL, verify FLARE is engaged.Ensure the autothrottle retards the thrust levers to idle by touchdown. |  |  |  |  |  |