# Takeoff Procedure

| PILOT FLYING | PILOT NOT FLYING |
| --- | --- |
| Advance thrust levers to approximately 40% N1. |  |
| Observe engine instruments stabilized and normal. |  |
| Press either TO/GA switch to advance the thrust levers to takeoff N1. |  |
| Verify mode annunciation. | Ensure thrust levers advance to takeoff N1. Observe mode annunciation. Note: In cases of extreme headwind, the thrust levers may not advance to full N1. In this case, manually advance the thrust levers as required. |
| Note: After takeoff thrust is set, the captain’s hand must be on the thrust levers until V1. |  |
| Hold light forward pressure on the control column, maintain directional control. | Monitor engine instruments. Verify oil pressure is not in the yellow band. |
| Verify 80 knots. | Call “80 KNOTS.” Verify that A/T annunciation changes to THR HLD by 84 knots. |
| Monitor airspeed, noting V1, and rotate smoothly at VR. | Call “V1,” and call “ROTATE” at VR. Monitor flight instruments. |
| When a positive rate of climb is indicated, call “GEAR UP” and continue rotation to takeoff pitch attitude. | Verify positive rate of climb. Position landing gear lever UP. |
| Check flight instrument indications. |  |

# After Takeoff Procedure

| PILOT FLYING | PILOT NOT FLYING |
| --- | --- |
| Maintain a minimum of V2 + 20 kts during initial climb. At light gross weight, a higher speed (up to V2 + 25) may be selected, to synchronize F/D pitch command and avoid objectionable body attitude. | Monitor engine instruments and cross–check flight progress. |
| Above 400 feet, call for appropriate roll mode, if required. Verify proper mode annunciation. | Select/verify roll mode. Verify proper mode annunciation. |
| Above 1,000 feet, call for N1 and flaps up maneuvering speed. Verify flight and thrust mode annunciations. | Select N1 and set flaps up maneuvering speed. Verify climb thrust is set. Verify proper mode annunciation. |
| When above minimum altitude for autopilot engagement, engage A/P. Verify flight mode annunciation. | Verify autopilot engaged. |
| Retract flaps on takeoff flap retraction speed schedule. | Position FLAP lever as directed and monitor flaps and slats retraction. |
| Call “AFTER TAKEOFF CHECKLIST” when flaps are up. | Position landing gear lever OFF, APU and engine start switches as required. Verify air conditioning and pressurization operating normally. Accomplish the AFTER TAKEOFF checklist. |
| Above 3,000 feet AGL, engage VNAV or select normal climb speed and verify annunciation. | Verify proper mode annunciation. |

# Takeoff Flap Retraction Speed Schedule

| T/O FLAPS | SELECT FLAPS | At & below 53,070 Kgs | Above: 53,070 Kgs up to 62,823 Kgs | Above 62,823 Kgs |
| --- | --- | --- | --- | --- |
| 15 | 5 1 UP | V2 + 15 170/F 190/F | V2 + 15 180/F 200/F | V2 + 15 190/F 210/F |
| 5 | 1 UP | V2 + 15 190/F | V2 + 15 200/F | V2 + 15 210/F |
| 1 | UP | 190/F | 200/F | 210/F |

- “F” – Minimum flap retraction speed, existing flap.

Note: Limit bank angle to 15 degrees until reaching V2 + 15.