# Climb and Cruise Procedure

| PILOT FLYING |  |  |  |  | PILOT NOT FLYING |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Position landing lights OFF passing through 10,000 feet. |  |
| Set altimeters to standard at transition altitude. |  |  |  |  |  |  |
| Approaching selected FMC cruise altitude, verify level off and proper mode/N1 limit annunciation. |  |  |  |  |  |  |
|  |  |  |  |  | Position center tank fuel pump switches OFF when both pump LOW PRESSURE lights illuminate. |  |
|  |  |  |  |  | During the last hour of cruise on all extended range (more than one hour from an adequate airport) flights, perform the Fuel Crossfeed Valve Check. |  |
| Set MCP altitude selector for descent. |  |  |  |  | Prior to top of descent, select and verify the planned arrival procedure on the FMC. |  |
| At top of descent point observe descent initiated and verify proper mode annunciation. |  |  |  |  |  |  |

Note: If a center tank LOW PRESSURE light(s) illuminates during takeoff or initial climb, the center tank pump(s) may remain on until the climb attitude is reduced and the light(s) extinguishes or workload allows for the pump(s) to be positioned OFF.

Note: When established in a level attitude at cruise, if the center tank contains usable fuel and the center tank pump switches are off, the center tank pump switches should be positioned ON again. If the center tank contains more than 1000 lbs/453 kgs, the center tank pump switches must be turned ON. Verify the LOW PRESSURE lights extinguish and position both switches OFF when both LOW PRESSURE lights illuminate.