BEFORE TAXI PROCEDURE

Call “BEFORE TAXI CHECKLIST.” 	C

Fuel HEAT switches ............................................. As needed F/O

Before takeoff with tank fuel temperature 0° C or below, set the fuel HEAT switches to ON for one cycle.

Fuel heat must be OFF for takeoff.

GENERATOR 1 and 2 switches ..................................... ON F/O

PITOT STATIC HEAT switches ...................................... ON F/O

WING ANTI-ICE switch ...................................... As needed F/O

ENGINE ANTI–ICE switches .............................. As needed F/O

PACK switches ................................................................. ON F/O

APU BLEED air switch ..................................................OFF F/O

Flight/Ground switch ..................................................... FLT F/O

Flap lever......................................................Set takeoff flaps F/O

Verify that the LE FLAPS EXT green light is illuminated.

Flight controls ..............................................................Check C

Make slow and deliberate inputs, one direction at a time.

Move the control wheel and the control column to full travel in both directions and verify.

Move the rudder pedals to full travel in both directions and verify.

TAXI