# Landing Roll Procedure

| PILOT FLYING | PILOT NOT FLYING |
| --- | --- |
| Ensure thrust levers at idle. |  |
| Disengage autopilot and control airplane manually. Verify autothrottle disengages automatically. | Verify autothrottle is disengaged. |
| Verify SPEED BRAKE lever (ground spoilers) - UP. | Verify SPEED BRAKE lever UP. Call out “SPEEDBRAKES UP.” If SPEED BRAKE lever is not UP, call “SPPEDBRAKES NOT UP.” |
| Verify proper autobrake operation. |  |
| Without delay, raise reverse thrust levers to the interlocks, hold light pressure until release, and then apply reverse thrust as required. | Monitor engine instruments and announce any engine limit being approached, exceeded or any other abnormalities. |
| At 60 knots, reduce reverse thrust to be at IDLE reverse when reaching taxi speed. | Call “60 KNOTS.” |
| Approaching taxi speed, slowly move the reverse thrust levers to the full down position. | Verify REVERSER UNLOCKED lights extinguished. |
| Prior to taxi speed, disarm the autobrake and continue manual braking as required. |  |

WARNING: After reverse thrust has been initiated, a full stop landing must be made.

# Taxi In Procedure

When clear of the active runway, the pilot taxiing positions the speed brake lever to the DOWN detent and the pilot not taxiing accomplishes the following:

SPEED BRAKE lever .......................................................Verify DOWN FLAP lever ................................................................................... UP

APU (if required) ..........................................................................START

PITOT STATIC HEAT switches ....................................................... OFF

FLT/GRD switch ............................................................................. GRD

ENGINE START switches ............................................................... OFF

LANDING/TAXI light

and STROBE light switches  ..............................As desired

FLIGHT DIRECTOR switches ........................................................ OFF

WEATHER RADAR ........................................................................ OFF

Transponder .............................................................................As desired

APU GENERATOR switches (if APU operating) ............................. ON